Lab 08: Setup up an Assembly Language Project in MPLAB

Lab Objective:

In this lab, you will learn about creating an assembly language project in MPLAB and embed it with proteus simulation.

Lab Description:

After installing MPLAB and Proteus in your PCs, follow the following steps shown below. These will show you how to make a project in MPLAB. Follow the step-by-step procedure as shown below to compile your code and embed it with Proteus.

- Select Project Wizard from Project tab.

- Click Next.

- Select PIC18f452 from the devices

- Select MSASM as assembly compiler.

- Click browse to set path for your new project.

- Select folder and file name for your project.

- Click Next.

- Select the following two files and add them in your project.

C:/Program Files/ Microchip/ Mpasm Suite/ p18f452.inc

C:/Program Files/ Microchip/ Mpasm Suite/ LKR / 18f452_g.lkr

- Click ‘New File’ icon to create a new file.

- Save that file with the extension .asm (you’ll have to type .asm)

- Right click Source File and click on Add Files…

- Select the .asm file that you saved earlier.

- Write a code in you .asm file and click on build all icon to build the project.

- In your Proteus, create the following circuit schematic diagram.

- In your proteus schematic, double click on PIC18F452 IC then select .hex file as program file.

- Now, simulate the circuit and observe the output.

Lab Task:

Perform the above mentioned steps, modify the above code and show toggling of LED in Proteus.